Security-Informed Safety Educational Resource

Student Worksheet

Student Name:

Student ID:

Episode Title: Smart Manufacturing Robot Cell Ransomware (Organisational Culture, Co-Design and Safe Recovery)

Associated Video: Episode 4 – Smart Manufacturing Robot Cell Ransomware

Purpose of this Worksheet

This worksheet helps you analyse how cyber security events can create, worsen, or reveal safety risks in cyber-physical systems.

You will examine:

- the system being protected

- the cyber security event

- the safety hazard

- the possible consequences

- appropriate incident response actions

- and suitable mitigation strategies.

The aim is not only to identify a cyberattack, but to understand how cyber events can influence physical processes, operational decisions, and safety outcomes.

# Scenario Summary

Current manufacturing environments rely more and more on integrated OT, industrial control systems, engineering workstations, HMIs, and industrial communication networks. These technologies enhance productivity, flexibility, and operational visibility. But at the same time, they also form dependencies on digital systems that could be interrupted in the event of cyber security incidents.

Ransomware is often viewed primarily as an availability problem. In manufacturing environments, however, the most significant safety risks frequently emerge during recovery rather than during the initial cyber incident itself. When operators lose trusted visibility of machine status, controller configurations, production recipes, or operational conditions, pressure to restore production can create circumstances in which unsafe assumptions are made.

This episode looks at a realistic smart-manufacturing robot cell affected by a ransomware incident. The attack does not alter robot motion or safety systems in a direct way. Rather, it impacts operational visibility and creates uncertainty regarding the trustworthiness of engineering and control systems. When production pressure increases, workers must decide if and how to restart the robot cell.

This episode illustrates a fundamental Security-Informed Safety principle:

It is not always safe to restart a system that has recovered from a cyber incident.

Safe recovery depends on effective coordination between cyber security, operations, maintenance, controls engineering, and safety personnel.

# System Context

What system is being examined?

# Cyber Security Event

What cyber security event, compromise, disruption, or security-driven action occurs?

# Safety Concern

What safety issue could result from the cyber event?

# Main Security-Informed Safety Lesson

What is the main lesson from this episode?

# Key Assets and Dependencies

Identify the important assets and dependencies in the scenario.

| Asset / System Component | Role in Normal Operation | Why It Matters for Safety |
| --- | --- | --- |

Examples of asset/system components: Engineering Workstation, HMI, Pump, Sensor, Controller, etc.

The assets can be mentioned in high-level or very detailed, depending on the purpose of your study.

Suggested assets to consider

- RC-101 Industrial Robot

- RC-CTRL-101 Robot Controller

- S-PLC-101 Safety PLC

- HMI-101 Human-Machine Interface

- EWS-101 Engineering Workstation

- Interlocked Access Gate

- Safety Light Curtain

- Emergency Stop System

- Manufacturing Execution System (MES)

- Production Monitoring Systems

- Data Historian

- Identity and Authentication Services

- Industrial Communication Network

- Operations Personnel

- Maintenance Personnel

- Safety Personnel

# Hazard–Threat Map

Complete the table below.

A threat is a cyber security-related event or condition.

A hazard is an unsafe system state or condition.

Potential harm is the consequence that could affect people, equipment, services, or the environment.

Students should distinguish between:

- the cyber incident

- the operational disruption

- the unsafe recovery condition

- the resulting hazard

- the potential harm

Remember that the ransomware incident itself is not the hazard.

The robot cell stops safely.

The hazard emerges during recovery when decisions must be made using incomplete or potentially untrustworthy information.

The hazard emerges from uncertainty rather than from direct cyber manipulation.

| Threat / Cyber Event | Affected Asset or Function | Resulting Hazard | Potential Harm |
| --- | --- | --- | --- |

# Cyber-to-Physical Escalation Pathway

Complete the pathway showing how the cyber event can become a safety issue.

Cyber Event: (Ransomware Incident)

Write your answer here

Affected Digital / Control Function: (Loss of Trusted Operational Information)

Write your answer here

Operational Deviation or Loss of Assurance: (Robot Cell Safe Stop and Production Pressure)

Write your answer here

Hazard: (Unsafe Restart Temptation and Incomplete Validation)

Write your answer here

Potential Harm: (Unexpected Robot Motion, Personnel Injury, Equipment Damage)

Write your answer here

Now write the pathway as one short paragraph.

Write your answer here

# Figure-Based Analysis

Answer the following questions.

# What does the figure show?

# Which component is most safety-critical?

# Which component is most security-critical?

# Where does the cyber-to-safety transition occur?

# Incident Response Task

You are part of the response team.

Your task is to respond to the incident while protecting safety.

Assume the ransomware incident has been contained and the robot cell is in a safe stopped condition. Production pressure is increasing, engineering workstations are being rebuilt, and some operational information remains unavailable. Your response actions should prioritise personnel safety, recovery assurance, incident containment, and safe recommissioning.

# First Response Actions

List the first five actions you would take.

| Priority | Action | Why This Action Matters |
| --- | --- | --- |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |
| 4 |  |  |
| 5 |  |  |

# Stakeholder Coordination

Who needs to be involved in the response?

| Stakeholder | What They Know | What They Should Do |
| --- | --- | --- |
| Cyber Security / SOC Team |  |  |
| OT / Engineering / Technical Team |  |  |
| Maintenance Technician |  |  |
| Operations Lead |  |  |
| Safety Representative |  |  |
| Manufacturing Manager |  |  |
| Vendor / External Support |  |  |

# Safety-Critical Decision

What actions should be taken before returning the robot cell to automatic operation following the ransomware incident?

What evidence should be required before approving automatic production restart?

Should production resume immediately, operate in reduced-speed commissioning mode, or remain stopped until all validation activities are completed?

# Mitigation Analysis

Analyse possible mitigations.

Classify each mitigation as one or more of the following:

- Preventive: reduces the likelihood of the incident.

- Detective: helps identify the incident or deviation.

- Protective: limits unsafe physical consequences.

- Recovery: supports safe restoration.

| Mitigation | Preventive | Detective | Protective | Recovery | Explanation |
| --- | --- | --- | --- | --- | --- |

Suggested mitigations to analyse

- Verification of incident containment

- Confirmation of authorised user accounts

- Inspection of robot-cell physical condition

- Validation of robot software integrity

- Validation of PLC software integrity

- Verification of Safety PLC diagnostics

- Gate-interlock testing

- Light-curtain testing

- Emergency-stop testing

- Reduced-speed recommissioning

- Cross-functional restart approval

- OT backup and recovery procedures

- Segmentation between IT and OT environments

- Recovery governance processes

- Co-design practices

# Security Controls vs Safety Controls

Security controls and safety controls do different things, but both may be needed.

Students should distinguish between controls intended to prevent or contain ransomware incidents and controls intended to protect personnel from hazardous robot motion. Some controls support both objectives, but safe recovery requires coordinated consideration of cyber security and functional safety.

Complete the table (add more rows, if necessary)

| Control | Security Control or Safety Control? | What Risk Does It Reduce? | What Happens If It Fails? |
| --- | --- | --- | --- |

Note: Some controls in this episode are cyber security controls, and some are safety controls. Students should identify which category each control belongs to and explain how they complement one another.

# Trade-Offs and Tensions

Cyber security and safety goals may sometimes create tension.

Identify one tension in the episode.

Examples may include:

- Production recovery versus safety validation.

- Rapid restoration versus recovery assurance.

- Availability versus trustworthiness.

- Operational pressure versus cautious recommissioning.

- Cyber security recovery versus safety verification.

- Business continuity versus safe restart.

- Individual decision-making versus cross-functional approval.

# What is the tension?

# Why is it difficult to resolve?

# How should it be managed?

# Security-Informed Safety Reflection

Answer the following questions.

# Why is this scenario not only a cyber security problem?

# Why is this scenario not only a safety problem?

# What does this episode teach about Security-Informed Safety?

# Group Discussion Task

In small groups, discuss the following.

Task

You are advising the manufacturing organisation following the ransomware incident described in the episode.

Prepare three recommendations:

- One recommendation to reduce the likelihood of the cyber event.

- One recommendation to detect or identify the developing hazard.

- One recommendation to reduce harm if the hazard occurs.

| Recommendation Type | Recommendation | Justification |
| --- | --- | --- |
| Reduce likelihood |  |  |
| Detect hazard |  |  |
| Reduce harm |  |  |

# Short Written Response

Write a short answer, approximately 150–250 words.

Explain how the episode demonstrates the relationship between cyber security, safety, and operational decision-making.

# Optional Extension Task

Choose one of the following.

Option A: Improve the Architecture

Suggest one change to the system architecture that would reduce cyber-to-physical risk.

Option B: Improve the Incident Response Plan

Suggest one change to the incident response process that would improve safety during recovery.

Option C: Improve Human Decision-Making

Suggest one way to support operators, engineers, clinicians, or other staff during degraded operation.

Option D: Improve Restart Governance

Propose one additional validation, approval, or assurance activity that should be completed before returning the robot cell to automatic production.

# Checklist Before Submission

Before submitting, check that you have:

- identified the cyber event

- identified the hazard

- distinguished hazard from harm

- completed the hazard-threat map

- completed the cyber-to-physical pathway

- proposed incident response actions

- analysed mitigations

- considered both security and safety controls

- explained the Security-Informed Safety lesson